2.02 Workshop – Model Set Up II: Boundary Conditions and Initial Conditions

For hydrodynamic, temperature, and water quality constituent simulations, initial conditions must be defined for each variable to be used at the start of the simulation. Boundary conditions must be specified for every time step. Depending on the variable, these boundary conditions may either be constant values (specified in either the control file or bathymetry file) or time series (specified in external files, with the names of these files defined in the control file). We will learn where this information is specified.

- Open the model control file, “w2_con_DeGray4.5.xlsm”.

- Open the manual: “CE-QUAL-W2_v4.5_Users_Manual_Part_3_InputOutputFiles_rev3.pdf”

- Hydrodynamic Boundary Conditions (BCs):

  - Locate the “BRANCH GRID” card on row 47 in the w2_con_DeGray4.5.xlsm control file and refer to the Branch Geometry section of Part 3 of the user’s manual on page 18.

  - The manual shows how different integer entries in the “branch grid” card control how the model reads in boundary conditions for computing hydrodynamics (head or flow).

  - UHS and DHS specify the upstream and downstream boundary conditions, respectively. If these are set to non-zero values, those will define constant conditions for the entire simulation. In our case, however, we see UHS = 0 and DHS = 0. Setting these values to zero tells the model to read time series from a flow file instead of using constant head boundary values.

- Branch External Upstream Head Elevation:

  - At the bottom of page 19 of Part 3 of the user’s manual, under Related Cards and Files, click the “Branch External Upstream Head Elevation File” link, which will take you to the section on page 381 that shows how to define this file.

  - In the control file, scroll to row 894 to see where the filename (EUHFN) is specified.

  - When you are finished, click the “external upstream head boundary condition” link to return to page 19.

- Branch External Downstream Head Elevation:

  - At the bottom of page 19 of Part 3 of the user’s manual, under Related Cards and Files, click the “Branch External Downstream Head Elevation File” link, which will take you to the section on page 381 that shows how to define this file. In the control file, scroll to row 897 to see where the filename (EDHFN) is specified.

- Branch Dependent Filenames:

  - In the control file, scroll to the “Branch Dependent Filenames” card on row 882.

  - There is a column for each branch. This indicates that inflow filenames must be specified for each branch. Note the file names given for QINFN (row 884) and QOTFN (row 887).

  - In your model directory, locate these files and open them in a text editor to inspect them. These files are the branch inflow and outflow files, respectively.

  - In the user’s manual, refer to the “Branch Inflow File” section on page 362 and the “Branch Outflow File” section on page 367.

  - Do these two files look like they should according to user’s manual?

- Hydrodynamic Initial Conditions (ICs):

  - Open Part 2 of the user’s manual, “CE-QUAL-W2_v4.5_Users_Manual_Part_2_Theory_rev3.pdf” and scroll to the section “Computation of Initial Water Surface Slope and Velocity Field for River Simulation” on page 21. For lakes and reservoirs, the initial conditions for velocity are usually set to zero, and the initial water surface elevations are set in the bathymetry file.

  - Initial Water Surface Elevations: In your model files directory, locate the bathymetry file, named bth1.csv and open it. Locate the line starting with “WSEL”. The values in this row show the initial water surface elevations for each segment of the model.

- Temperature BCs (inflow): The temperature boundary condition will be based on how the hydrodynamic BC is set up. If there is a flow BC, there will be a temperature inflow file. If there is a head BC, there will be an external temperature file for both upstream and downstream. We are not worried about the head boundaries nor the temperature files associated with those. Look at the row 885 to see what the temperature inflow file (TINFN) is named. Look this up in the file directory to see what it looks like.

- Temperature ICs and downstream BC:

  - Refer to the Initial Conditions (INIT CND) section of Part 3 of the user’s manual, on page 22. The temperature ICs can be defined three ways: 1. Isothermal, 2. Single vertical profile, or 3. Vertical profile at each segment.

  - In the control file, locate the “T2I” field of the “INIT CND” (initial conditions) card on row 65. We see that the initial temperature of -1 tells the model to read a single vertical profile from an external file, which initializes the temperature at every segment using this vertical profile. The filename is specified in the VPRFN field of the “Waterbody Dependent File names” card on row 875. Check the file name of this vertical profile and open the corresponding file from the file directory.

  - Refer to the “Vertical Profile File” section of Part 3 of the user’s manual on page 391 to see the specifications for the format of this file. As you can see, initial constituent concentrations as well as initial temperatures are specified using this file.

  - The downstream temperature BC is based on these ICs. At every computation timestep, the downstream temperature BC is pulled from the previous timestep.

- Constituent Concentrations BCs (inflow) and downstream BC:

  - The constituent concentration inflow BC is similar to the temperature BC settings. Refer to the “Constituent Initial Concentration (CST ICON)” section of the user’s manual on page 143.

  - In the control file, scroll to the “CST - Concentration State variables and initial conditions” card on row 401. The initial condition values for each of the active constituents are listed in column H.

- Other inputs: Flows, temperature, and concentrations can be impacted by other inputs, such as atmospheric deposition (ATMDEPFN; row 874), distributed flows (QDTFN; row 888), and precipitation (PREFN; row 891)

  - Check the file directory to see which of these files are present. You should see files for atmospheric deposition and distributed flows. Open the files in a text editor and inspect them.

  - Atmospheric deposition is only a mass loading. It is not connected to a fluid flow. Distributed inflow, temperature, and concentration are connected with fluid flows.

  - In the control file, scroll to the “Atmospheric Deposition” card on row 397 to view controls for the atmospheric deposition. What do these say?

  - Scroll to the DST Tributaries card on row 288 to view controls for the distributed flows. Does this model contain distributed tributaries?

- Time Series Outputs:

  - In the control file, locate the “TSR Plot” card on row 355. We will add a second time series output for WQ constituents. Make the following changes:

    - Change the value of NITSR in Cell C358 to 2. This tells the model to output two time series.

    - Change the value of TSR SEG in Cell C362 to 28 and enter a new value of 31 in Cell D362. The model will output data for segments 28 and 31.

    - Change the value of TSR LAYER in Cell C363 to 2. Instead of outputting data at the surface layer (0), it will output from layer 2.

    - Enter a new value of 16 for TSR LAYER in Cell D363.

- Run the Model:

  - Save the control file

  - Scroll to the top of the control file and locate the button located on or near cell B8.

  - Click the button to export the settings to a CSV file, which will be read by the model.

  - Run the pre-processor program and inspect the output. Check for error and warning messages. If there are no errors and no major issues are indicated in the warnings file, you are ready to run the model

  - Run the CE-QUAL-W2 model. It will read the CSV file you just created and will run the simulation.

  - After the simulation completes, verify that two TSR files were created by the model. These should be named “tsr_1_seg28.csv” and “tsr_2_seg31.csv”.

  - Open these files in Excel and verify that water temperature and the active constituents were output as expected.

  - Verify that values were output from the model start time to the model end time, as specified in the “TIME CON” card on row 27 of the control file.